| Name: | Shahzaman Dhillon |
| --- | --- |
| Reg No: | 23-NTU-CS-1273 |
| Section: | BSAI 5th |
| Subject | Embedded IOT |
| Submitted To: | Sir. Nasir Mehmood |

HOMEWORK-1 (AFTER MID)

Question-1

(Part A):

1. What is the purpose of WebServer server(80); and what does port 80 represent?
WebServer server(80); creates a web server object on the ESP32 that listens for incoming HTTP requests.
Port 80 is the default port for HTTP web traffic, so any browser can access the ESP32 without specifying a port number.

2. Explain the role of server.on("/", handleRoot); in this program.
This line defines a route handler.
When a client accesses the root URL ("/") of the ESP32 web server, the function handleRoot() is automatically called to generate and send the response.

3. Why is server.handleClient(); placed inside the loop() function? What will happen if it is removed?
server.handleClient(); continuously checks for and processes incoming client requests.
It must be inside loop() so the ESP32 can respond to users in real time.
If it is removed, the web server will stop responding, and the browser will not receive any page or data.

4. In handleRoot(), explain the statement: server.send(200, "text/html", html);

- 200 → HTTP status code meaning request successful

- "text/html" → content type of the response

- html → the webpage content being sent
This statement sends an HTML webpage to the client’s browser as a successful HTTP response.

5. What is the difference between displaying last measured sensor values and taking a fresh DHT reading inside handleRoot()?

- Last measured values:
Uses previously stored sensor data → faster response, no sensor delay.

- Fresh DHT reading inside handleRoot():
Reads the sensor every time the page loads → more accurate but slower and may cause sensor timing issues.

PART B

Working of ESP32 Webserver-Based Temperature and Humidity Monitoring System

The ESP32-based temperature and humidity monitoring system uses a DHT sensor, an OLED display, and a web server to monitor environmental conditions both locally and remotely through a web browser. The system works through the following stages:

- First, the ESP32 connects to a Wi-Fi network using SSID and password. After successful connection, the router assigns an IP address to the ESP32.

- Then, the web server is started using port 80. When a user enters the ESP32 IP address in a browser, the request is received and handled by the server.

- A button is used to read temperature and humidity from the DHT sensor. When the button is pressed, the ESP32 reads the sensor values and shows them on the OLED display.

- The webpage is created using HTML code stored in a string. Sensor values are added dynamically inside the HTML code so updated data is shown.

- A meta refresh tag is used in the webpage to automatically reload the page after some seconds, so new values appear without manual refresh.

Common Issues and Solutions

- Wi-Fi not connecting → check SSID and password

- Webpage not loading → ensure server.handleClient() is in loop

- Wrong sensor values → check DHT type and wiring

- Slow response → avoid delay() in web server code

QUESTION NO.2

PART A

1. Role of Blynk Template ID in an ESP32 IoT project

The Blynk Template ID uniquely identifies the device template created on the Blynk Cloud. It links the ESP32 firmware to the correct cloud dashboard, widgets, and data structure.
It must match the cloud template because the Blynk server uses this ID to map incoming data to the correct virtual pins, widgets, and automation rules. If it does not match, the device will fail to connect or display data incorrectly.

2. Difference between Blynk Template ID and Blynk Auth Token

| Feature | Blynk Template ID | Blynk Auth Token |
| --- | --- | --- |
| Purpose | Identifies device template | Authenticates a specific device |
| Scope | Same for all devices using the template | Unique for each device |
| Created in | Blynk Cloud template settings | Generated per device |
| Function | Defines layout and widgets | Allows secure cloud connection |

3. Why DHT22 code gives incorrect readings with a DHT11 sensor

DHT11 and DHT22 use different data formats and timing protocols. When DHT22 code is used with DHT11, the ESP32 misinterprets the received data, resulting in wrong values.

Key difference:

- DHT11 → Integer values, lower accuracy

- DHT22 → Floating-point values, higher resolution

4. What are Virtual Pins in Blynk and why are they preferred?

Virtual Pins are software-defined pins used for sending and receiving data between the ESP32 and Blynk Cloud. They do not correspond to physical GPIO pins.

They are preferred because:

- They allow cloud communication without hardware pin limitations

- Multiple widgets can use the same Virtual Pin

- Data remains functional even if GPIO mapping changes

5. Purpose of using BlynkTimer instead of delay()

BlynkTimer enables non-blocking execution, allowing multiple tasks (sensor reading, cloud communication, UI updates) to run smoothly.
Using delay() blocks the processor, causing:

- Wi-Fi disconnections

- Missed Blynk updates

- Unresponsive devices

Therefore, BlynkTimer ensures stable, real-time IoT operation.

Part B:

Explain the complete workflow of interfacing ESP32 with Blynk Cloud to display temperature and humidity values.

- First, a Blynk Template is created on the Blynk Cloud.

- Datastreams are added for temperature and humidity, usually using Virtual Pins such as V0 and V1.

- Widgets like labels or gauges are connected to these datastreams.

- The Template ID, Template Name, and Auth Token are added in the ESP32 program.

- These details allow the ESP32 to connect securely with the Blynk Cloud.

- The DHT sensor (DHT11 or DHT22) is connected to the ESP32 and configured correctly in the code.

- Incorrect sensor configuration can cause wrong readings.

- The ESP32 reads temperature and humidity values from the sensor.

- These values are sent to the Blynk Cloud using Blynk.virtualWrite().

- A BlynkTimer is used to send data periodically without blocking the program.

- The cloud receives the data and updates the widgets on both the Blynk mobile app and web dashboard in real time.

- This system allows users to monitor temperature and humidity remotely using their mobile phone or browser.